Analogue and Digital conversions

# Analogue Signals

An analogue signal contains all data in a continuous varying wave. The vibrations created by this wave are sensing by the ear an recognized as sound.

Analogue storage device devices tend to have poor fidelity meaning they deteriorate slightly each time they are played.

If a wave is vibrating at 500 oscillations per second, it is vibrating at 500 hertz

1 oscillation = 1 hertz

Digital Signals

A series of pulses, on;s or off;s (binary math). A digital signal must be transferred back into an analogue signal before it can be heard.

In all digital recording technology the goal is to create a recording with very high fidelity (very high similarly between the original signal and the reproduced signal) and perfect reproduction (the recording sounds the same every signle time you play it).

The conversion ids done by a device called an analogue-to-digital converter (ADC)

To play back the music, the stream of nubers is converted back to an analogue wave by a digital-to-analogue converter (DAC)

When you sample the wave wtig an analogue-didital converter, you have to control two variable:

-the sampling rate (x-axis) – controls howmany samples ate taken per second

-the sampling precision (y-axis) – controls howmany different gradation

(quantization levels) are possible when taking the sample

In the following figure, let’s assume that the sampling rate is 1000 per second and the precision is 10

You can see that as the rate ans precision increase, the fidelity (the similarity between the original wave and the DAC’s output) improves.

In the casae of CD sound, fidelity is an important goal, so the sampling rate is 44,100 samplest per second and the number of gradations is 65,537.

AT this level, the output of the DAC so closely matches the original waveform that the sound is essentially “perfect” to most human ears.

CD storage capacity

One thing about the CD’s sampling rate and precision is that it produces a lot of data.

IN a CD player, there asre 44,100 16-bit samples of the audio per second, and there are two streams for sereophonic playback,

This makes for a “sampling

Rate” that is called “stereo 16-bit 44.1 kHz’ (k=1000)

There are two sound streams being recorded (one for each of the speakers on a stereo system)

A CD can store up to 74 minutes of musicm so the total amount of digital data must be stored a a CD is

Sample rate * precision * # of channels * time (sec)

44.100 samples/channel/second * 2

bytes/sample * 2 channels * 74 minutes * 60

seconds/minute = 783,216,000 bytes

| # Source | Bit depth | Sampling rate |
| --- | --- | --- |
| Cd audio (stereo | 16 bits | 44.1 kHz |
| DAT (stereo) | 16 bits | 48 kHz |
| DAT (extended play) | 16 bits | 32 kHz |
| MniDisc | 16 bits | 48 kHz **compressed |
| MiniDisc | 16 bits | 32 kHz ** extended play |
| DAT XT20 | 20 bits | 48 kHz |
| DVD Audio | 24 bits | 96 kHz |
| Pro MO disk | 24 bits | 96 kHz |

- Calculate the memory raquired to save 2 hours of DVD (6 chan) audio

- the memory required to save 74 minutes of DAT audio

- calculate the momory required to save 74 minutes of mini disk audio

q